.~

.. ~

ROMAN BLINDS

:MEASURING THE WINDOW AND CALCULATING THE FABRIC

1.	Decide were you want the blind to go -inside or outside the frame, of the window.

2.	Measure finished width and length, where you want the blind to go. It is a good idea to write down these measurements, as it is not always the size of the window. .

3.	To the length measurement add -

40cm to each drop of fabric if your blind is less than 100cm long

50cm to each drop of fabric if your blind is less than 150cm long

60cm to each drop of fabric if your blind is less than 200cm long

This extra fabric allows for top and bottom hems, and the pleats to hold-

the rings and the rods if you want them.

DON'T BE MEAN, IT IS MUCH HARDER TO ADD EXTRA LENGTH IF YOU ARE SHORT .

4.	Divide window width by the fabric width. If window width is less than fabric width, you will require one drop of fabric. If it is more you will require two drops of fabric.

5.	When using more than one width of fabric it is a good idea to avoid the join at the centre.

6.	Pattemed fabric will need to be matched, so allow more. The first drop wi1l be the length required, and the next drop will be the length and the amount of the pattern match.

7.	To get the amount required, multiply the length of each drop by the number of drops, to give you the total length required, and divide by 100 to convert to metres.

8.	For the side seam of the blind you will need to allow 3cm for each side, for a hem and if you are joining two drops together another 3cm for seam a11owance. DON'T BE MEAN, THAT IS THE LEAST AMOUNT REQUIRED.

9.	Lining is optional. The requirement for lining is the width of the blind, and the length of each drop less 15cm.

OTHER REQUIREMENTS NEEEDED

40mm by 19mm timber for the headboard, the width of the blind

30mm by 10mm timber for the baseboard of the blind, the width of the blind less 2cm.

I use treated Pine. Don’t use any of the composite boards, as the moisture around the window makes them swell and disintegrate and also they are likely to bend.

Plastic rings	 (20 to 25cm across the blind)

(20 to 30cm apart up the blind)

Multiply number of pleats, by number of rings across the width to give you total number required. Most shops have a chart to work these out, but do allow a few extra in case you break one or two sewing them on.

Screw eyes – one for each row of plastic rings.

Blind cord – length of blind plus width by number of rows of rings. Again the shops have a chart to help you.

Cord end, with a hole in it big enough to thread your cord through.

Cleat and screws.

Screws to attach the blind to the wall.

Thread to sew blind.

OPTIONAL

You can put rods in your blinds.

Fibreglass rods or fine doweling. These give your blind a much more tailored look but are not necessary on a narrow blind. They are more important if you are making a very wide blind.

Bracket and screws to attach the blind.

Please bring your own pins, tape measure, scissors, Bernina bobbin if you have one, and any other sewing aids that will help you.

Bernina machines are available for your use, but you are most welcome to bring your own if you prefer.

It is a help to label scissors by tying something to the handle, as they are the most common things to get mixed up.

Please bring your lunch. The college will provide tea and coffee.